# Alteon WebSystems Training

Basic Switch Configuration

Laboratory Guide

## Copyright 2001

This document contains proprietary information, which is the property of Alteon WebSystems, Inc.  This document may not, in whole or in part, be copied, photocopied, reproduced, translated, or reduced to any electronic medium or machine-readable form without prior consent, in writing, from Alteon WebSystems, Inc.

### IMPORTANT

ALTEON WEBSYSTEMS, BEING BOTH A MANUFACTURER AND USER OF SOFTWARE, HAS A RESPONSIBILITY AND A LIABILITY TO ENSURE THAT NO SOFTWARE IS ILLEGALY COPIED OR DISTRIBUTED.

YOUR PRESENCE IN USING THIS MATERIAL IMPLIES YOUR ACCEPTANCE OF THE TRAINING REQUIREMENTS THAT NO SOFTWARE OR SUPPORTING MATERIAL WILL BE COPIED FROM ANY COMPUTER, DISKS, AND/OR CD-ROM SUPPLIED FOR THIS PROGRAM.

Introduction

This laboratory module introduces the student to the installation, configuration, and trouble shooting of various real-world implementations of the Alteon switches.

Objective

Following the lecture portion of this course, the student will be called upon to install, configure and troubleshoot basic Alteon switch configurations.  The desired result of this hands-on portion of the class is to build command line interface proficiency and to display the skill sets necessary to correctly place and configure an Alteon switch to suit various network scenarios.

## Critical Skills

- Understand the system and switch requirements for installation, configuration, and operation

- Familiarization with the use of the switch interface

- Display competency uploading and downloading switch code from a TFTP server

- Complete the configuration, operation and troubleshooting of the following switch implementations:

- Server Load Balancing (SLB)

- Web Cache Redirection (WCR)

- Virtual Router Redundancy Protocol (VRRP)

- Firewall Load Balancing (FWLB)

- Any new configuration as designed by the instructor (for additional experience)

- Understand switch troubleshooting techniques and maintenance features

Laboratory length

Approx. 9 cumulative hours (depending on user skill set)

## Reference material

Class Lecture

Alteon Technical Training Guide

Release 8.3 Command Reference

Release 8.3 Application Guide

#### LABORATORY GUIDE

Program:		Alteon WebSystems – Switch Configuration and Operation

## Prerequisite:  	Be familiar with the Command Reference, the Application Guide and the latest switch documentation prior to completing this laboratory guide.

Prepared by:  	Alteon WebSystems, Inc.

Release Date:             April 2001

Instructor:       	_________________________________________________________

Company:		_________________________________________________________

Date:		            _________________________________________________________

Student:		_________________________________________________________

Company:		_________________________________________________________

Comments:     	_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

TABLE OF CONTENTS

Exercise 1:    Basic Switch Configuration

Exercise 2a:  Cut and Paste Switch Configuration

Exercise 2b:  Upload and Download to TFTP

Exercise 2c:   Change Switch Code Using Image Files

Exercise 3:    Server Load Balancing (SLB) Switch Configuration

Exercise 4:  Virtual Router Redundancy Protocol (VRRP) Configuration

Exercise 5:    Switch Troubleshooting

Exercise 6:  Web Cache Redirection (WCR) Switch Configuration

Exercise 7:  Firewall Load Balance (FWLB) Switch Configuration

Appendix A:  Common useful screen dumps

Appendix B:  Complete configuration dumps for each lab

Appendix C:  Serial Download Instructions

EXERCISE 1:  BASIC SWITCH CONFIGURATION

OBJECTIVE:

Configure and test a basic switch configuration.

ASSIGNMENT:

Configure three IP interfaces on the ACEdirector switch as shown in the diagram

below.  After the interfaces have been properly configured and enabled, establish

IP forwarding to allow data connectivity between the interfaces.  Test your physical

connectivity by pinging through to each remote device from each node.  If the pings

are successful, use the client to retrieve a web page from each of the server computers

proving HTTP functionality.

BASIC SWITCH NETWORK DIAGRAM:

##### Figure 1:  Basic Switch Configuration

##### Note 1:  If any student would like to attempt the configuration without the instructions,

##### close the lab book at this time.  Since it is important to stay on time for the course,

##### use your best judgment to know when you are having difficulty and take the appropriate

##### course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise at

this time.

- Connect the cables to the switch and set up the PC’s IP addresses as noted in Figure 1.

- Open HyperTerminal and set the terminal configurations as follows:

Baud rate = 9600

Data bits =  8

Parity = none

Stop bits = 1

- Save the terminal configuration file.

- Activate the terminal session with the switch by pressing <enter>.

- Log into the switch, enter the admin password – admin.

- Set the switch to factory default.

/boot/conf factory……...	short form /b/co f

reset…………………….	/b/r, reset the switch

y………………………...	confirms reset, pressing enter reboots the switch

n…………………………after reboot, do not run set up configuration

- Log into the switch using the admin password.

- Configure the interfaces for the switch, refer to Figure 1.

/cfg/ip/if 1

mask 255.255.255.0……	always enter the mask 1st to ensure proper broadcast address

addr 10.X.11.1…………	where X is your instructor-designated station number

y………………………..	disable the use of BOOTP

en………………………	to enable the interface (if 1)

/cfg/ip/if 2/mask 255.255.255.0/addr 10.X.1.1/en

/cfg/ip/if 3/mask 255.255.255.0/addr 10.X.3.1/en

apply……………………	to apply the configuration

save…………………….	save the configuration

y………………………..	confirm saving to FLASH

y………………………..	save to FLASH and change to the active config block

Note: During this lab a default gateway does not need to be set.  The default

gateway will need to be set in the real environments.  The command

structure is /cfg/ip/gw to set the default gateway.

- Turn on IP forwarding on the switch

/cfg/ip/frwd

on…………………. this allows data to flow between the switch interfaces

(this is enabled by default in 8.x code)

- Reset the switch

/b/r………………..this resets the switch

y………………….this is to confirm the reset

(this reset is necessary for the bootp disable command to be saved)

- Check the current configurations of your switch

/cfg/dump…………this displays your configuration information

- Ping the remote devices on the network from your CLI to confirm Layer 1

connectivity

ex:  ping 10.X.1.32

- Using a browser on the client to retrieve a web page from each server to confirm

that HTTP is operational

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE!!!

### STOP

(unless provided additional instructions by the instructor)

SUMMARY:

The purpose of this lab was to familiarize you with console connection setup and

how to save the HyperTerminal configurations.  Once connected, you were shown

the steps necessary for IP interface and IP forwarding configuration.  Finally, after

completing your configuration changes, you were instructed on how to enable, apply,

and save your settings for future use.  An acronym to help remember to save your

work is EASY.  (E = Enable, A = Apply, S = Save, Y = Yes, to confirm the save)

QUESTIONS:

- What is the command set used to configure IP on an interface of a switch?

- What is the command set to boot your switch back to factory defaults?

- What is the command set to activate, employ, and store a configuration change?

- What protocol allows your client’s web browser access to the web servers?

5.  What is the default admin password?

EXERCISE 2a:  CUT AND PASTE SWITCH CONFIGURATION

OBJECTIVE:

To successfully edit and load the switch configuration using copy and paste

ASSIGNMENT:

Take the active configuration file and modify it by copying a command string onto

your clipboard and then pasting it into the HyperTerminal interface and saving it

as your new active configuration.

FYI:  Depending on the telnet client being used (e.g. XTERM, HyperTerminal, etc.),

- be aware of the length of the lines transmitted and (2) the application can

insert end-of-line characters that can affect the configuration download operation.

Example: If a line is greater than 132 characters, most telnet clients will wrap

the line.  These line wraps can affect the way the configuration file is transmitted

and saved.

##### Note 1:  If any student would like to attempt the configuration without the instructions,

##### close the lab book at this time.  Since it is important to stay on time for the course, use

##### your best judgment to know when you are having difficulty and take the appropriate

##### course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise at

this time.

- Save the switch coniguration as a text file:

  - Type verbose 0 on the switch, this puts the switch in quiet mode

  - In Hyperterminal, click on Transfer  Capture Text

  - Lable the file SW1.TXT and save it in the C:\temp directory

  - Click on Start

  - /cfg/dump

  - In Hyperterminal, click on Transfer  Capture Text  Stop

  - /cfg/verbose 2

- Edit the switch configuration file, SW1.TXT, stored in the C:/temp directory using Notepad.  Add the following line after the “if 1” command line:

/cfg/ip/if 2/mask 255.255.255.0/addr 10.X.1.1/broad 10.X.1.255/ena

where X is your instructor-designated station number

- Copy the command line you just typed onto the clipboard

Highlight /cfg/ip/if 2/mask 255.255.255.0/addr 10.X.1.1/ vlan 1/ena

Then go to the Edit menu and select Copy

- Save the modified file in the Temp directory as SW1-SER.TXT

- Log into the switch using HyperTerminal.  At the main switch menu, type:

- verbose 0……….This command sets the level of information displayed, (0 = quiet)

- At the switch prompt, paste the modified line into HyperTerminal

- Go to the Edit menu and select Paste, then apply and save your changes as follows:

- apply/save y

- Once the changes have been saved, type the following: 
verbose 2………to get out of quiet mode

- Dump the switch configuration to the screen and verify the edited line was applied:
/cfg/dump

Note:  You can do this for selected command lines or a whole switch configuration!

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE!!!

### STOP

(unless provided additional instructions by the instructor)

SUMMARY:

This lab allowed you to develop the skill to drag and drop a series of commands into

the terminal interface.  This lab displayed the possibility for a “canned” switch

configuration to be implemented from a saved text file.  You were also made aware

of some of the pitfalls of this type of configuration, such as accidentally inserted

end-of-line characters which can cause the switch to misinterpret the commands and

not respond accordingly.

QUESTIONS:

- Identify the range of Verbose settings and relate their level of information.

- Which menu contains the Copy and Paste selection options?

- What does the dump command do?

4.   What command set would you use to view current switch configuration?

EXERCISE 2b:  UPLOAD AND DOWNLOAD TO TFTP

OBJECTIVE:

Become comfortable uploading and downloading a file to a TFTP server

ASSIGNMENT:

Modify the command line used in exercise 2a and save your changes.  Second,

locate the TFTP server in your network and log the IP address.  Next, upload the

modified file to the TFTP server.  Finally, the file will be modified one more time

at the TFTP server, saved, and then downloaded back to the switch.

Remember to always verify the configuration transmitted correctly to the switch

or to the TFTP server when uploading and downloading switch configuration files.

Figure 2:   TFTP Configuration

##### Note 1:  If any student would like to attempt the configuration without the instructions,

##### close the lab book at this time.  Since it is important to stay on time for the course, use

##### your best judgment to know when you are having difficulty and take the appropriate

##### course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise

at this time.

1.    Select the TFTP server icon on the desktop to start the service

2.    Enter the IP address of the TFTP server ____________________  
       Check the root directory of the TFTP server (this is where files will loaded from and sent to)

3.    Save the file as SW1-PUT to the TFTP server

/cfg/ptcfg……………… used to upload the active configuration to a TFTP server

Enter IP address of TFTP server:

Enter name of file on TFTP server: (ex. SW1-PUT)

4.    Check that the file (SW1-PUT) was saved on the TFTP server by looking in the root directory of the TFTP server

5.   Open the file SW1-PUT using Notepad.  You may need to word-wrap the text

to see the lines of interest

6.    Modify the command line in SW1-PUT to make the IP address for if 2 to 10.X.4.1 and the broadcast address to 10.X.4.255

7.    Save the file as SW1-GET.txt

8.  Go to the switch CLI prompt and enter the following commands:

/cfg/gtcfg……………… used to replace active configuration with downloaded file

Enter IP address of TFTP server:

Enter name of file on TFTP server: (ex. SW1-GET.txt)

apply/save
y

9.  Check to see if your configuration changes have been saved

/cfg/ip/cur

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE!!!

### STOP

##### (unless provided additional instructions by the instructor)

##### SUMMARY:

##### This lab should build your confidence using the ptcfg and the gtcfg commands

##### to upload and download text files to a TFTP server.

##### QUESTIONS:

##### What are the two pieces of information that must be known to upload or

##### download a file?

##### What command other than dump can be used to confirm a change has been

##### made?

3.    If you have trouble connecting to your TFTP server what should you check?

##### EXERCISE 2c:  CHANGE SWITCH CODE USING IMAGE FILES

##### OBJECTIVE:

To teach you how to download switch code from the TFTP server and load it onto one of your image blocks and also to be able to boot to either stored image on your switch

ASSIGNMENT:

Obtain the proper switch code for the hardware being used in the lab.  A guide for

determining which code type to use is available below.  Start your TFTP server.

Log into the switch and establish which image you are booted on.  Download your

switch code from the TFTP server to the unused image block. Reboot the switch

to the other image and ping to test connectivity.

##### Note 1:  If any student would like to attempt the configuration without the instructions,

##### close the lab book at this time.  Since it is important to stay on time for the course, use

##### your best judgment to know when you are having difficulty and take the appropriate

##### course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise

at this time.

1.  Obtain the appropriate switch code for the appropriate switch (e.g., AD2, AD3,

AD4, 180, and 180e) from the TFTP server.  The file was placed for you into

the C:/Program Files/Cisco Systems/Cisco TFTP Server folder.  Normally you

would have to obtain the code from Alteon WebSystems and load it into the

TFTP server yourself.

AD2 and 180 series typically have a code name of <filename).180.

Example: 5237_mp.180

AD3 and 180e series typically have a code name of <filename>.180e

Example: 6055_mp.180e

AD4 and 184 series typically have a code name of <filename>.184

Example: 8039_mp.184

2.  Enter the switch code filename:  ____________________________________

3.  Start the TFTP server and enter its IP address __________________________

4.  Log into the switch and access the boot menu

/b………………………..go to the boot menu and look at the options

cur……………………...review the current images

Which is the active image?  image1 or image2 (circle one)

5.  Verify which image block you want to update and boot the switch to the other

image (ex. Update image2, boot to image1)

/b/image

image X…………..where X is the image that you want to boot into

/b/r

6.  Ping the TFTP server to verify connectivity

7.  Update the switch with a TFTP switch code file to the designated image block.

/boot/gtimg

image X…………….where X is the image you want to revise from TFTP

Enter IP address of TFTP server:

Enter name of file on TFTP server:

Confirm download: y

At the prompt, the successful download process is denoted by a series of

repeated  ..C..C..C..  characters.  Confirm successful download by reviewing

the TFTP screen and the switch status.

8.   Confirm that you want to use the new image on the next boot. Reboot the switch to use the new image, then confirm the image status.

/b/r

/b/cur

- Ping the TFTP server and other remote devices from the switch to verify connectivity.

- The reverse operation can be done to save the software image to the TFTP server by using the /boot/ptimg command. Try that now on your own, but call the file another name so the original is not overwritten on the TFTP server.

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE!!!

STOP

(unless provided additional instructions by the instructor).

SUMMARY:

You learned each series of switch uses it’s own version of switch code.  You are

starting to utilize your TFTP server as a switch maintenance tool by employing

it for code updates.  You understand that there are multiple images on your switch

available for you to update and boot into.  The code itself must be obtained from

Alteon WebSystems and will need to be manually placed in the TFTP server by

the switch administrator.

QUESTIONS:

- What command set would you use to download switch code to image1

on the switch?

- What command did you use to review which image is currently in use?

- What should you do after downloading new code to your switch?

4.  You are updating your switch code. The filename you find in your TFTP

server is 6055_mp.180e, will this code work on your AD4 switch? Why

or why not?

5.  What command set would you use to switch images and reset the switch?

EXERCISE 3:  SLB SWITCH CONFIGURATION

OBJECTIVE:

Configure and test a Server Load Balance (SLB) switch configuration.

ASSIGNMENT:

Set the switch back to factory defaults.  Set up real servers and add them to a group.

Use Round Robin as your metric and TCP for your Health Check.  Configure the

virtual IP.  Enable the ports and define its state (client/server).  Turn off Spanning

Tree to reduce wait time.  Enable server load balancing.

SLB NETWORK DIAGRAM:

Figure 3: SLB Network Diagram

##### Note 1:  If any student would like to attempt the configuration without the instructions,

##### close the lab book at this time.  Since it is important to stay on time for the course, use

##### your best judgment to know when you are having difficulty and take the appropriate

##### course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise at

this time.

- Connect your client PC to the serial interface of the switch.

- Activate the terminal session with the switch (i.e., <enter>).

- Enter the admin password (i.e., admin)

- Check the configuration, version, and image being used in the switch.

Type device:	_______________________________________

Software version:	_______________________________________

Flash image:	_______________________________________

Note:  In an operational environment, check the current documentation, notices

and/or updates regarding the code release and any known issues.  If it is a later

version of code, contact your Alteon representative for a newer code release.

- Set the switch to factory default

/boot/conf factory………short form /b/co f

reset……………………../b/r, reset the switch

y…………………………confirm reset

After reboot:

n…………………………do not run set up

- Configure Interface 1 with IP information and disable BOOTP.

/cfg/ip/if 1

mask X.X.X.X…………..example 255.255.255.0

addr Y.Y.Y.Z………….	example 10.X.11.1

y…………………………disable the use of BOOTP

en……………………….	to enable the interface (if 1)

apply……………………	to apply the configuration

save…………………….	save the configuration

y………………………..	confirm saving to FLASH

y………………………..	save to FLASH and change to the active config block

- Connect the cables to the switch and configure the IP addresses on the servers and clients as noted in Figure 4.

- Configure the real servers.

/cfg/slb/real 1…………………...short hand /c/sl/r 1

rip <real server IP address>…….example: rip 10.X.11.32

en

../………………………………..to take you up 1 level in the menu hierarchy.

real 2…………………………...short hand r 2

rip <real server IP address>

en/apply

save

y

../cur……………………………...verify your current configurations

- Add the real servers to a group

/cfg/slb/group 1………………..or type .. or /c/sl/gr 1

add 1…………………………..add real server 1 to group 1

add 2…………………………..add real server 2 to group 1

metric round………………….select round robin for the metric

health tcp……………………..TCP is the default

apply/save

y

../cur

- Configure the virtual IP

/cfg/slb/virt 1……………….…where 1 is the virtual interface #. (or /c/sl/v 1)

vip <IP address>……………....example:  vip 10.X.11.11

en

service  <http>………………..assign the protocol to the VIP and real server group

apply/save

y

../cur

11.   Enable the client and server ports on the switch

/cfg/slb/port 1………………….short hand /c/sl/p 1

client e…………………………...activate client and enable the port

/cfg/slb/port 2/client e

/cfg/slb/port 7

server………………………….activate server

e

/cfg/slb/port 8/server e

12.  Turn SLB on and STP off

/cfg/slb/on

/cfg/stp/off

apply/save

y

/b/r……………………………..necessary reset for STP change to occur

y

At this point, if it has not happened already, the switch should report the slb:  real server 10.X.11.X operational.

Date Time NOTICE slb: real server 10.X.11.32:80 operational

Date Time NOTICE slb: real server 10.X.11.33:80 operational

13.  Check the current SLB configuration

/cfg/slb/cur

14.  Check connectivity to all four remote devices from the switch using ping

15.  Verify SLB is working from the client to the server using the web browser

interface.

From the client (PC 31 or 35):

Open a web browser (Netscape or MS Internet Explorer)

Type http://<IP address of VIP>		example http://10.X.11.11

You should see a response noting you have reached Server Y

(where Y = Server 32 or 33).

16.  Press CTRL-F5 to refresh the screen display.  You may notice that your

browser toggles between the web servers.  This will probably occur because

of the metric you chose for your virtual server (round-robin).  Press CTRL-F5

a few more times to generate some data to track.

Note: You may also see that you are remaining on one server, which can be

misleading from the user perspective.  To verify you are load balancing, review

the switch statistics.

17.  Verify SLB is working from the statistics menu in the switch

/stats/slb/virt 1……………….shortcut  /st/sl/v 1,response is in appendix A

18.  Go to the browser and press CTRL-F5 several more times to generate traffic.

19.  Return to the switch UI and note changes to the switch statistics.

(For hot key use, press “” key to get /stats/slb/virt 1)

*Note the total session statistics for each real server and for the VIP during SLB activity.

20.  Clear the session table and repeat testing SLB  (steps 15 through 19)

/oper/slb/clear…………………..this clears the session table

21.  Save the working SLB configurations to PC 31 (Client)

In the switch, go to the console menu, select Transfer.  Select Capture

text mode.  Provide a directory path and file name for the saved configurations

(ex. C:\temp\SLB SW1.txt). Press Start to begin the capture.

At the switch prompt, type

verbose 0		verbose and the number 0, a blank screen will appear

/cfg/dump

verbose 2		to return to the screen display

At the end of this screen print, go to console menu and STOP the transfer

(Transfer->Capture text-> Stop.  Verify the file was saved to the designated

location. (open it with notepad)

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE!!!

STOP

(unless provided additional instructions by the instructor).

SUMMARY:

In this exercise, you discovered how to define your virtual IP that will be advertised

to the world.  You also set the real IP addresses of each server that will participate

in the load balancing process.  Next, you added the servers with like content to a

group and assigned the load balancing to that server group .  You set the client/server

relationships on each active port.  Finally, you enabled SLB on your switch and then

verified that load balancing was occurring by generating some traffic and using various

statistics commands to track the traffic load.

QUESTIONS:

- Explain the difference between a real and virtual IP’s?

- Which metrics were available to you when setting up your server group?

- How did the command /oper/slb/clear affect switch statistics, be specific?

4.  What is the default health check for your server group?

EXERCISE 4:  VRRP - TWO SWITCH CONFIGURATION

OBJECTIVE:

Configure and test the VRRP switch configurations.

ASSIGNMENT:

Configure the switch for SLB use ICMP for your health checks.  Configure and

enable VRRP for default gateway redundancy.  Set up switch priority to determine

ownership.  Understand the terms: master, backup, owner, sharing, and tracking.

##### VRRP NETWORK DIAGRAM:

If1 10.X.11.1

Vip 10.X.11.11

If1 10.X.11.2

Vip 10.X.11.11

10.X.11.32

10.X.11.33

Figure 4: VRRP Network Diagram

##### Note 1:  If any student would like to attempt the configuration without the instructions, close the book at this time.  Since it is important to stay on time for the course, use your best judgment to know when you are having difficulty and take the appropriate course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise now.

1.  Configure SLB on Switch 1 (from Exercise 3)

2.  VRRP configuration on Switch 1.

/cfg/slb/group 1

healt icmp……………….change health check to icmp

/cfg/vrrp/on

/cfg/vrrp/vr 2

n………………………….respond no to decrementing bridge priority (if using AD2)

vrid 2……………………must be an *even* number in earlier code

addr X.X.X.X…………..	set equal to the VIP (ex. 10.X.11.11)

if 1/en

/cfg/stp/on

/cfg/stp/brg/hello 2/fwd 4/mxage 6

(Verify port configurations)

/cfg/slb/port 1/client e

../port 3/client e/server e………..vrrp port must have client and server enabled
../port 7/server e

../port 8/server e

apply/save

y

/b/r ……………………to reset switch and activate spanning tree

3.  Set up the network as noted in the VRRP Network diagram

Note: There may be some error messages on the switch configuration screen.  This will occur as you reconfigure the switches.

4.  VRRP configuration of switch 2.

Move console connection to Switch 2.

/b/co f……………Set the switch to factory default

/b/r		Reset the switch

y			Confirm switch reset

n			No to ‘set up” facility

/cfg/ip/if 1/mask 255.255.255.0/addr 10.X.11.2/en/apply/save

y …………………turn off bootp

y

y

/cfg/slb/on

/cfg/vrrp/on

n…………………respond no to decrementing bridge priority (if using AD2)

/cfg/stp/on

/cfg/stp/brg/hello 2/fwd 4/mxage 6

/cfg/slb/sync/peer 1

addr 10.x.11.1…………..address of “if 1” on first switch

ena

/oper/slb/sync

y

10.x.11.1…………..enter address of “if 1” on first switch

y……………………this operation will fail for now since Switch 1 is not done yet

apply/save

y

/b/r

5.  Continue configuration of Switch 1. Move console connection back to Switch 1. Both switches must have the same version of Web OS in order for VRRP synchronization to work.

/cfg/slb/sync/peer 1

addr 10.x.11.2…………..address of “if 1” on second switch

ena

/oper/slb/sync

y

At this point, make Real Server 1 the primary on Switch 1 and backup on Switch 2.

Make Real Server 2 the backup on Switch 1 and the primary on Switch 2.

With the console connection still on Switch 1:

/cfg/slb/group 1/rem 2/

/cfg/slb/real 1/backup 2/apply/save

y

../cur

6.  Continue configuration of Switch 2.

Move the console connection to Switch 2.

/cfg/slb/group 1/rem 1

/cfg/slb/real 2/backup 1/apply/save

y

../cur

7.  Test the VRRP configuration:

From PC31 (client):

Ping 10.X.11.11………….. Ping the VIP

Open the web browser, http://10.X.11.11 and access the servers (PC32 or PC33)

From PC35 (client):

Ping 10.X.11.11

Open the web browser, http://10.X.11.11 and access the servers (PC32 or PC33)

Access Switch 1 CLI:

/info/slb/real <server number>..….Note the status of the health check for the servers.

Note: The health checks for both real servers and the virtual server should report “up” status.

/info/vr

What is the priority? ________

Is this switch the master or backup? _________

/i/stp

Which ports are forwarding? __________________

Which ports are blocking? ____________________

/st/vr

How many VRRP advertisements have been received? ____________________

How many VRRP advertisements have been sent out? ____________________

Access Switch 2 CLI:

/info/slb/real <server number>…..…Note the status of the health check for the servers.

Note: The health checks for both real servers should report “up” status.

/info/vr

What is the priority? ________

Is this switch the master or backup? _________

/i/stp

Which ports are forwarding? __________________

Which ports are blocking? ____________________

/st/vr

How many VRRP advertisements have been received? ____________________

How many VRRP advertisements have been sent out? ____________________

8.   With the serial cable plugged into Switch 2, POWER OFF Switch 1.

Note the operational messages for the virtual router, etc. on Switch 2.

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Access Switch 2 CLI:

/info/vr

What is the priority? ________

Is this switch the master or backup? _________

/i/stp

Which ports are forwarding? __________________

Which ports are blocking? ____________________

/st/vr

How many VRRP advertisements have been received? ____________________

How many VRRP advertisements have been sent out? ____________________

9.  POWER ON Switch 1 with the serial cable plugged into Switch 2.

Note any operational messages for the real servers, etc. on Switch 2.

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Note any operational messages for the real servers, etc. on Switch 1.

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

10.  Access Switch 2 CLI:

/info/vr

What is the priority? ________

Is this switch the master or backup? _________

/i/stp

Which ports are forwarding? __________________

Which ports are blocking? ____________________

/st/vr

How many VRRP advertisements have been received? ____________________

How many VRRP advertisements have been sent out? ____________________

11.  From PC31, Press “CTRL-F5” a few times and note the Switch

information.

Can you access the real servers through the VIP? ____________________

What are the Switch 1 statistics? ___________________________________

What are the Switch 2 statistics? ___________________________________

(/stats/slb/virt 1)

12.  Access Switch 1 CLI:

/info/vr

What is the priority? ________

Is this switch the master or backup? _________

/i/stp

Which ports are forwarding? __________________

Which ports are blocking? ____________________

/st/vr

How many VRRP advertisements have been received? ____________________

How many VRRP advertisements have been sent out? ____________________

13.  Save the working VRRP switch configurations to PC 31 (Client)

One of the ways to save the configuration, especially if you will be cutting and pasting from the configuration is the following:

In the switch screen, go to the Hyperterminal menu, select Transfer.

Select Capture text mode.  Provide a directory path and file name for the saved configurations (ex. C:/temp/VRRP SW 1.txt; C:/temp/VRRP SW 2.txt).

Press Start to begin the capture.

At the switch prompt, type

verbose 0		verbose and the number 0, a blank screen will appear

/cfg/dump

verbose 2		to return to the screen display

At the end of this screen print, go to Hyperterminal menu and STOP the transfer (Transfer->Capture text-> Stop).  Verify the file was saved to the designated

location.

14.  Save Switch 2 configuration as noted above.

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE !!!

STOP

(unless provided additional instructions by the instructor).

SUMMARY:

This exercise should have taught you the basics of VRRP.  How VRRP uses a

priority scheme to determine each switches role.  The way these switches

know who has highest priority is through inter-switch multicast messages.

Tracking can be used to periodically check switch health to determine which

machine should assume the master role.  Sharing allows load balancing to

occur between the switches even though one of the machines is acting in the

backup role, it will still process traffic.

QUESTIONS:

1.  Explain in detail what benefits VRRP gives your network?

2.  What is the VRID and what does it affect?

3.  What feature must be turned on for the Active-Active scenario to function

properly?

4.  What is the default priority when configuring VRRP?

5.  What is the difference between an owner and a renter?

EXERCISE 5:  SWITCH TROUBLESHOOTING

OBJECTIVES:

To familiarize yourself with the common commands to check switch status and to

ensure successful switch maintenance activities

ASSIGNMENT:

Learn to use the diff command to view changes before saving them.  Review the CLI

commands to check critical switch functions (such as port speed, STP configuration,

SLB configuration, etc). Cultivate the ability to spot errors in your configuration.

Acquaint yourself with the techniques to gather switch statistical data for troubleshooting.

##### Note 1:  If any student would like to attempt the configuration without the instructions,

##### close the lab book at this time.  Since it is important to stay on time for the course, use

##### your best judgment to know when you are having difficulty and take the appropriate

##### course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise at

this time.

- Using the diff command.

After applying configurations, use the “diff” command to review changes. This command is available universally just like apply and save.

Ex:  /cfg/ip/if 2/mask 255.255.255.0/addr 10.X.1.1/mask 255.255.255.0/en/diff

- Review port configuration (e.g., speed 10/100/1000, etc.)

/cfg/port <num>			            ex. /cfg/port 1

/cfg/port <num>/cur			choose ports in use and review options

/cfg/port <num>/fast			choose ports in use and review options

/cfg/port <num>/fast/speed		choose ports in use and review options

/cfg/port <num>/fast/mode		choose ports in use and review options

/cfg/port <num>/fast/fctl		choose ports in use and review options

/cfg/port <num>/fast/auto		review available option

- Review STP configuration (forwarding/blocking status)

/cfg/stp/cur/info/stp

- Review errors, overflow and buffer statistics

/stat/port #/ether

/stat/port #/if

/stat/port #/link

/stat/port #/maint

5.  Perform the following commands using the current SLB configuration

(Note: Some of the commands you did previously are noted in the table

below for reference purpose).

| CLI COMMAND | COMMENT |
| --- | --- |
| LAYER 2 – useful CLI commands |  |
| /info/sys | Provides system information, IP, software version, etc. |
| /info/link | Provides port link status |
| /info/fdb/dump | Provides forwarding database information, VLANs, etc. |
| /info/arp/dump | Provides arp table information |
| /info/ip | Provides IP information |
| /cfg/dump | Provides switch configuration dump |
| /stat/port <num>/ <ether/if/link> | Provides port statistics |
| /stat/port <num>/maint | Provides port maintenance statistics |
| /stat/if <num> | Provides identified interface information |
| /stats/mp | Provides management processor utilization information |
| ## LAYER 4 – useful CLI commands |  |
| /info/slb | Provides SLB information |
| /info/dump | Provides dump of current switch information |
| /cfg/slb/cur | Provides SLB current configuration review |
| /stat/slb/rip <real-server-num> | Provides statistics by real IP (RIP) |
| /stat/slb/group <real-server-group #> | Provides useful group information |
| /stat/slb/virt <virtual-server-num> | Provides virtual services information (e.g., VIPs, etc.) |
| /stat/slb/maint | Provides SLB maintenance statistics |
| /stats/dump | Provides switch statistics information |
| /info/slb/sess | Provides SLB session information |

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE!!!

STOP

(unless provided additional instructions by the instructor).

SUMMARY:

One of the most important skill sets to cultivate is the ability to quickly gather

information about your switch for effective troubleshooting and maintenance.

Because Alteon switches work on several layers and provides services outside

of the normal layer 2 switch duties, this skill becomes even more critical.

QUESTIONS:

- What command would you use to check the Spanning Tree status?

- If  I wanted statistics for port 3 what command would I use?

- When I try to save my configuration I receive a message telling me my

broadcast address for interface 2 is incorrect.  What command could

I use to view my recent changes?

4.  What command would you use to view arp table information?

EXERCISE 6:  WEB CACHE REDIRECTION (WCR)

OBJECTIVE:

Configure and test the WCR switch configuration.

ASSIGNMENT:

Treat the web caches like any other server in a SLB configuration.  Identify them as

real servers, add them to a real server group, and attach then to a virtual interface.

Once SLB is configured, build some filters to be placed on the INGRESS ports of

the switches to redirect your incoming HTTP traffic to the web caches.  Once the

configuration is complete, use your information and statistics menus to gather session

data.

##### WCR NETWORK DIAGRAM:

Figure 5:  WCR Network Diagram

##### Note 1:  If any student would like to attempt the configuration without the instructions, close the book now!  Since it is important to stay on time for the course, use your best judgment to know when you are having difficulty and take the appropriate course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, begin the exercise below.

- Connect the cables to the Student-Side Switch according to Figure 5

- Configure WCR on Switch 1.

/boot/conf f
y

/b/r

y

After switch reboots, enter password and decline to use setup

/cfg/ip/if 1

mask X.X.X.X…………………..ex. mask 255.255.255.0

addr X.X.X.X…………………...ex. addr 10.X.11.2

en/apply

y………………………………….disable bootp when prompted

save

y

/cfg/slb/real 1

rip X.X.X.X…………………….example: rip 10.X.11.32 (the web cache server)

en

../real 2/rip X.X.X.X/en……….the address of the other server (10.X.11.33)

../group 1/add 1/add 2

metric hash…………………….hash or minmisses can be used

health tcp/apply/cur

- Create filters.

/cfg/slb/filt 2/sip any/dip any/proto tcp/sport any

dport http…………..…………..or dport 80

actio redir/rport 80/group 1/en

/cfg/slb/filt 224/sip any/dip any/proto any/act allow/en

- Apply filters to ports.

../port 2…………………………example: assign filters to ports 2 and 3

add 2/add 224/filt e

../port 3/add 2/add 224/filt e

- Turn on SLB and verify status.

/cfg/slb/on/apply

save

y

/cfg/slb/cur

- Test WCR setup.

On PC31:

Using the browser, log into a few web sites, press “CTRL-Refresh” many times.

Return to the switch UI and note changes to the switch statistics.

/stats/slb/group 1		(For hot key use, press “” key)

- Review the web caching statistics on the web caching systems.

From the browser on PC31 or PC35:

http://<IP address>:81/cgi-bin/cachemgr.cgi

Note:  using port 81 to redirect port 80 traffic

- On Web caches, example of items to review.

IP cache stat and content

HTTP Header stats

General runtime information

All cache objects

In memory/in transit

Server side network read <size> histogram

Traffic and resources counter

Cache digest and ICP blob

Cache utilization

Full histogram stats

Client-side active requests

##### Log into the switch and review the switch statistics.

/cfg/slb/cur

/info/slb/dump

/stats/slb

- Return to the switch UI and the cache servers and note changes

- Repeat steps 7 through 11 numerous times and review WCR activities.

To make filter changes take effect immediately, clear the session binding

tables for the port.

/oper/slb/clear…………….for each port

- Save the working WCR switch configurations to PC 31 (Client)

In the switch, go to the Hyperterminal menu, select Transfer.

Select Capture text mode.  Provide a directory path and file name for the

saved configurations (ex. C:/temp/WCR sw 1.txt). Press Start to begin the

capture.

At the switch prompt, type

verbose 0

/cfg/dump

verbose 2

At the end of this screen print, go to Hyperterminal menu and STOP the transfer

(Transfer->Capture text-> Stop).  Verify the file was saved to the designated location.

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE !!!

STOP !!!

(unless provided additional instructions by the instructor).

SUMMARY:

This lab should have given you a chance to feel more confident about building and

defining an SLB configuration and it is your first exposure to creating filters and

applying it to a particular switch ingress port.

QUESTIONS:

1.  Explain the frame modifications (NAT’s) that occur during WCR operations?

2.  True or false, filter 224 is a global filter designed to receive frames that did not

trigger previous filters and applies the default allow or deny operation?

3.  What must you do after creating a filter to make it operational on your switch?

4.  For persistence, which metrics would you choose for WCR operations?

EXERCISE 7:  FIREWALL LOAD BALANCING (FWLB) SWITCH CONFIGURATION

OBJECTIVE:

Configure and test the FWLB switch configurations.

ASSIGNMENT:

You will configure each firewall in sections to facilitate troubleshooting as you

work from the lower networking layers (Layers 2-3) to the upper networking layers

(Layers 4-7).  Verify SLB is working properly.  Verify connectivity to the internal FW

interfaces.  Verify connectivity from clients to servers.  Ensure redirection filters and

static routes are configured correctly.

##### FWLB NETWORK DIAGRAM:

Figure 6:  Firewall Load Balancing (FWLB)

##### Note 1:  If any student would like to attempt the configuration without the instructions, do not turn the page.  Since it is important to stay on time for the course, use your best judgment to know when you are having difficulty and take the appropriate course of action (e.g. review the instructions).

Note 2:  For any student that would like to use the instructions, turn the page and begin the exercise.

- Connect the cables to the switches as noted in the figure above.

- Confirm the proper static routes are set up in the FWs (Alteon Switches).

Alteon Switches are to be set up as a router to simulate the FWLB feature of the switch in a “2 switch” firewall configuration.

3.  Confirm the static routes were properly configured.

Configure Firewall 1

- Configure 2 interfaces

/cfg/ip/if 1/mask 255.255.255.0/addr 10.X.1.32/en
y
/cfg/ip if 2/mask 255.255.255.0/addr 10.X.2.32/en
apply/save
y

- Configure 2 static routes

/cfg/ip/route/add 10.X.12.0 255.255.255.0 10.X.2.1 2
/cfg/ip/route/add 10.X11.0 255.255.255.0 10.X.1.1 1
apply/save
y

CONFIGURE FIREWALL 2

- Configure 2 interfaces

/cfg/ip/if 1/mask 255.255.255.0/addr 10.X.3.33/en
y
/cfg/ip if 2/mask 255.255.255.0/addr 10.X.4.33/en
apply/save
y

- Configure 2 static routes

/cfg/ip/route/add 10.X.12.0 255.255.255.0 10.X.4.1 2
/cfg/ip/route/add 10.X11.0 255.255.255.0 10.X.3.1 1
apply/save
y

CONFIGURE FWLB ON SWITCH 1 (Dirty-Side)

8.  Set switch 1 to factory default

/boot/conf factory/

r…………………………confirm reset

After switch reboots, enter password and decline to use setup

9.  Configure 3 interfaces

/cfg/ip/if 1/mask 255.255.255.0/addr 10.X.11.1/en

y……………..disable bootp

/cfg/ip/if 2/mask 255.255.255.0/addr 10.X.1.1/en

/cfg/ip/if 3/mask 255.255.255.0/addr 10.X.3.1/en

apply/save

y

y

../cur

10.  Turn on IP forwarding.

/cfg/ip/frwd/on

11.  Define your static routes for each network.

/cfg/ip/route

add 10.X.2.0 255.255.255.0 10.X.1.32

add 10.X.4.0 255.255.255.0 10.X.3.33

apply/save

y

../cur

12.  Check Layers 2 and 3 connectivity.

ping 10.X.11.1

ping 10.X.1.1

ping 10.X.3.1

ping 10.X.1.32

ping 10.X.2.32

ping 10.X.3.33

ping 10.X.4.33

13.  Turn SLB on

/cfg/slb/on

14.  Configure the real servers

/cfg/slb/real 1/rip 10.X.2.1/en

/cfg/slb/real 2/rip 10.X.4.1/en

apply/save

../cur

15.  Add the real servers to a group

/cfg/slb/gr 1/add 1/add 2/metric hash/health icmp/apply/save

y

../cur

16.  Define the filters

This is the filter to the outbound link (currently the client side).

/cfg/slb/filt 100/sip any/dip 10.X.11.0/dmask 255.255.255.0

action allow……………..default is allow

en

/cfg/slb/filt 224/sip any/dip any/action redir/en/apply/save

y

17.  Apply the filters to the client’s ingress port

/cfg/slb/port 1/add 100/add 224/filt e/apply/save

y

../cur

18.  Verify the configuration is correct

/cfg/dump

19.  Reboot the switch

/b/r

CONFIGURE FWLB ON SWITCH 2 (Clean-Side)

20.  Set Switch 2 to factory default

/boot/con f

reset

confirm reset

After switch reboots, enter password and decline to use setup

21.  Configure 3 interfaces.

/cfg/ip/if 1/mask 255.255.255.0/addr 10.X.12.1/en

y

/cfg/ip/if 2/mask 255.255.255.0/addr 10.X.2.1/en

/cfg/ip/if 3/mask 255.255.255.0/addr 10.X.4.1/en/apply/save

y

22.  Turn on IP forwarding.

/cfg/ip/frwd/on

23.  Define your static routes for each network.

/cfg/ip/route

add 10.X.1.0 255.255.255.0 10.X.2.32

add 10.X.3.0 255.255.255.0 10.X.4.33

apply/save

y

../cur

24.  Check Layers 2 and 3 connectivity.

ping 10.X.12.1

ping 10.X.12.34

ping 10.X.12.35

ping 10.X.2.1

ping 10.X.4.1

ping 10.X.1.32

ping 10.X.2.32

ping 10.X.3.33

ping 10.X.4.33

25.  Turn SLB on

/cfg/slb/on

26.  Configure the real servers

/cfg/slb/real 1/rip 10.X.1.1/en/../real 2/rip 10.X.3.1/en/apply/save

y

../cur

27.  Add the real servers to a group

/cfg/slb/gr 1/add 1/add 2/met ha/health icmp/apply/save

y

../cur

28.  Define the filters

This is the filter to the server side of the FW.

/cfg/slb/fi 100/sip any/dip 10.X.12.0/dmask 255.255.255.0

action allow………………..default is allow

en

/cfg/slb/fi 224/sip any/dip any/act redir	/en/apply/save

y

29.  Apply the filters

/cfg/slb/port 1/add 100/add 224/ filt e

/cfg/slb/port 3/add 100/add 224/ filt e/apply/save

y

../cur

30.  Verify the configuration is correct

/cfg/dump

Note:  For troubleshooting, verify your configurations against the FWLB

switch configurations in Appendix B of this exercise manual.

31.  Reboot the switch

/b/r

32.  Test and verify the configuration is working

Test from the client to the server (using the web browser).

http://10.X.12.34 or http://10.X.12.35 to access the web servers.

You should see a response noting you have reached the selected server.

33.  From the switch CLI (for both switches), view the statistics:

/i/slb

/i/stp

/stats/slb/gr 1

34.  Save the working URL switch configurations to PC 31 (Client)

In the switch screen, go to the Hyperterminal menu, select Transfer.

Select Capture text mode.  Provide a directory path and file name for the

saved configurations (ex. C:/temp/FWLB SW 1.txt; C:/temp/FWLB SW 2.txt).

Press Start to begin the capture.

35.  At the switch prompt, type

verbose 0		verbose and the number 0, a blank screen will appear

/cfg/dump

verbose 2		to return to the screen display

At the end of this screen print, go to Hyperterminal menu and STOP the transfer (Transfer->Capture text-> Stop).  Verify the file was saved to the designated

location.

36.  Save Switch 2 configuration as noted above.

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE !!!

STOP

(unless provided additional instructions by the instructor).

SUMMARY:

After completing this lab, you should feel confident building and applying filters

on the ingress ports of the switches to redirect traffic through the firewalls.  Static

routes should have been added to the firewalls so traffic destined through the

sandwich will know which interface to travel through.  Also, note the way the hash

algorithm modifies frames according to the type of load balancing configured.

QUESTIONS:

1.  When used with a VIP for SLB, the hash algorithm uses only what portion of

a frame?

2.  What is the default gateway for a group of real servers when SLB is used in

conjunction with your firewall load balancing configuration?

3.  Along with the redirection filter, another filter should be built to handle a

specific type of traffic.  Identify this filter type and the traffic it was built

accommodate?

4.  What type of health checking is used with FWLB in this exercise?

5.  Define the static routes that need to be placed on each firewall and the

command you would use to implement them?

APPENDIX A

The following are examples of the most common commands to obtain

informational screen dumps.

COMMON SCREEN DUMPS

1.  info/sys

System Information at 13:23:22 Tue Feb 13, 2001

ACEdirector 3

sysName:

sysLocation:

Last boot: 13:18:13 Tue Feb 13, 2001 (reset from console)

MAC address: 00:60:cf:46:4f:f0    IP (If 1) address: 10.2.12.1

Hardware Revision: C

Hardware Part No: B02_5A-D_7B-A

Software Version 8.3.22-SSH (FLASH image2), active configuration.

2.  info/link

------------------------------------------------------------------

Port   Speed    Duplex     Flow Ctrl      Link

----   -----   --------  --TX-----RX--   ------

1      100     full     yes    yes       up

2      100     half      no     no       up

3      100     half      no     no       up

4    10/100  any     yes    yes      down

5    10/100  any     yes    yes      down

6    10/100  any     yes    yes      down

7      100     half      no     no       up

8      100     half      no     no       up

3.  info/fdb/dump

MAC address     VLAN  Port  State  Referenced ports

-----------------  ----  ----  -----  ----------------

00:60:cf:42:40:60     1    1    FWD    1

4.  info/arp/dump

IP address    Flags    MAC address    VLAN Port Referenced ports

--------------- ----- ----------------- ---- ---- ----------------

10.2.11.2        P    00:60:cf:45:96:d0    1       1-8

5.  info/ip

Interface information:

1: 10.2.11.2,       255.255.255.0,   10.2.11.255,     vlan 1, up

Default gateway information: metric strict

Current IP forwarding settings: OFF, dirbr disabled

Current local networks:

Current RIP settings:

OFF, update 30

split horizon

BGP Information:

OFF, id 0.0.0.0

BGP Peer Information:

6.  /cfg/dump

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken 13:24:48 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:4f:f0

/cfg/sys

bootp dis

/cfg/ip/if 1

ena

addr 10.2.12.1

mask 255.255.255.0

broad 10.2.12.255

/cfg/ip/if 2

ena

addr 10.2.2.1

mask 255.255.255.0

broad 10.2.2.255

/cfg/ip/if 3

ena

addr 10.2.4.1

mask 255.255.255.0

broad 10.2.4.255

/cfg/ip/route

add 10.2.1.0 255.255.255.0 10.2.2.32

add 10.2.3.0 255.255.255.0 10.2.4.33

/cfg/slb

on

/cfg/slb/real 1

ena

rip 10.2.1.1

/cfg/slb/real 1/layer7

/cfg/slb/real 2

ena

rip 10.2.3.1

/cfg/slb/real 2/layer7

/cfg/slb/group 1

metric hash

health icmp

add 1

add 2

/cfg/slb/filt 100

ena

action allow

dip 10.2.12.0

dmask 255.255.255.0

/cfg/slb/filt 224

ena

action redir

group 1

rport 0

/cfg/slb/port 1

filt ena

add 100

add 224

/cfg/slb/port 3

filt ena

add 100

add 224

/

/script end  /**** DO NOT EDIT THIS LINE!

7.  info/stp

------------------------------------------------------------------

Current Root:            Path-Cost Port Hello MaxAge FwdDel Aging

8000 00:60:cf:42:40:60        5     1    2     20     15    300

Number of topology changes:        8

Time since last topology change:    0 days, 00:54:03

Parameters:  Priority  Hello  MaxAge  FwdDel  Aging

32768      2      20      15     300

Port  Priority   Cost    State     Designated Bridge       Des Port

1     128        5   FORWARDING  8000-00:60:cf:42:40:60   32769

2     128       10   FORWARDING  8000-00:60:cf:45:96:d0   32770

3     128       10   FORWARDING  8000-00:60:cf:45:96:d0   32771

4     128        0    DISABLED

5     128        0    DISABLED

6     128        0    DISABLED

7     128       10   FORWARDING  8000-00:60:cf:45:96:d0   32775

8     128       10   FORWARDING  8000-00:60:cf:45:96:d0   32776

8.  /cfg/slb/cur

Current Layer 4 settings:

ON, direct disabled, matrix enabled, grace disabled

imask 255.255.255.255, pmask 255.255.255.255

mnet 0.0.0.0, mmask 255.255.255.255

Current health scripts settings:

Current real servers settings:

1: 10.2.11.32, enabled, name , weight 1, tmout 10, maxcon 200000

backup none, inter 2, retry 4, restr 8

remote disabled, proxy enabled, submac disabled

2: 10.2.11.33, enabled, name , weight 1, tmout 10, maxcon 200000

backup none, inter 2, retry 4, restr 8

remote disabled, proxy enabled, submac disabled

Current virtual servers settings:

Current filters:

2: enabled, invert disabled

sip any, dip any

proto tcp, sport any, dport http

action redir, group 1, rport 80, fwlb disabled

log disabled, cache enabled, proxy enabled

ack disabled

option disabled, tos 0 0 0

tcp no flags enabled

224: enabled, invert disabled

sip any, dip any

proto any

action allow

log disabled, cache enabled

ack disabled

option disabled, tos 0 0 0

Current port settings:

1: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

2: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt  enabled, filters: 2 224

3: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt  enabled, filters: 2 224

4: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

5: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

6: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

filt disabled, filters: empty

Current Global SLB settings:

OFF, dns enabled, ttl 60, local disabled, one disabled, alway disabled

http enabled, usern disabled, mincon 1024, inter 1, weight 1, geo enabled

Current remote site settings:

Current Lookup settings:

lookups disabled, dname none

Current internet network preference settings:

Current sync settings:

filt enabled, ports enabled, prios enabled, pips disabled

Current peer switch settings

9.  /info/slb/dump

Real server state:

1: 10.2.11.32, 00:10:a4:bb:15:e7, vlan 1, port 7, health 4, up

2: 10.2.11.33, 00:10:a4:b9:b5:92, vlan 1, port 8, health 4, up

Virtual server state:

Redirect filter state:

2: dport http, rport http, group 1, health tcp, backup none

real servers:

1: 10.2.11.32, backup none, up

2: 10.2.11.33, backup none, up

Port state:

1: 0.0.0.0

filt disabled, filters: empty

2: 0.0.0.0

filt  enabled, filters: 2 224

3: 0.0.0.0

filt  enabled, filters: 2 224

4: 0.0.0.0

filt disabled, filters: empty

5: 0.0.0.0

filt disabled, filters: empty

6: 0.0.0.0

filt disabled, filters: empty

7: 0.0.0.0

filt disabled, filters: empty

8: 0.0.0.0

filt disabled, filters: empty

10.  /info/ip/route/dump

Interface information:

1: 10.2.11.2,       255.255.255.0,   10.2.11.255,     vlan 1, up

Default gateway information: metric strict

Current IP forwarding settings: OFF, dirbr disabled

Current local networks:

Current RIP settings:

OFF, update 30

split horizon

BGP Information:

OFF, id 0.0.0.0

BGP Peer Information:

Destination         Mask          Gateway        Type      Tag    Metr If

--------------- --------------- --------------- --------- --------- ---- --

10.2.11.0       255.255.255.0   10.2.11.2       direct    fixed           1

10.2.11.2       255.255.255.255 10.2.11.2       local     addr            1

10.2.11.255     255.255.255.255 10.2.11.255     broadcast broadcast       1

127.0.0.0       255.0.0.0       0.0.0.0         martian   martian

224.0.0.0       224.0.0.0       0.0.0.0         martian   martian

224.0.0.0       240.0.0.0       0.0.0.0         multicast addr

255.255.255.255 255.255.255.255 255.255.255.255 broadcast broadcast

11.  /info/vlan

VLAN                Name               Status Jumbo  Ports

----  -------------------------------- ------ -----  ----------------

1     Default VLAN                       ena    n   1-8

12.  /cfg/port # <between 1 and 8>/cur

Current Port 8 configuration: enabled, PVID 1

13.  info/dump

System Information at 13:27:31 Tue Feb 13, 2001

ACEdirector 3

sysName:

sysLocation:

Last boot: 13:18:13 Tue Feb 13, 2001 (reset from console)

MAC address: 00:60:cf:46:4f:f0    IP (If 1) address: 10.2.12.1

Hardware Revision: C

Hardware Part No: B02_5A-D_7B-A

Software Version 8.3.22-SSH (FLASH image2), active configuration.

Last 10 syslog message information:

Feb 13 13:18:29 NOTICE  console: incorrect password

Feb 13 13:18:32 NOTICE  console: admin login on Console

Feb 13 13:18:47 NOTICE  system: rebooted (reset from console)

Feb 13 13:19:01 NOTICE  slb: real server 10.2.1.1 operational

Feb 13 13:19:02 NOTICE  slb: real server 10.2.3.1 operational

------------------------------------------------------------------

Port   Speed    Duplex     Flow Ctrl      Link

----   -----   --------  --TX-----RX--   ------

1      100     full     yes    yes       up

2    10/100     any     yes    yes      down

3      100     full     yes    yes       up

4    10/100     any     yes    yes      down

5    10/100     any     yes    yes      down

6    10/100     any     yes    yes      down

7      100     full     yes    yes       up

8      100     full     yes    yes       up

9     1000     full     yes    yes      down

------------------------------------------------------------------

Current Root:            Path-Cost Port Hello MaxAge FwdDel Aging

8000 00:60:cf:46:4f:d0       10     7    2     20     15    300

Number of topology changes:        3

Time since last topology change:    0 days, 00:08:45

Parameters:  Priority  Hello  MaxAge  FwdDel  Aging

32768      2      20      15     300

Port  Priority   Cost    State     Designated Bridge       Des Port

1     128        5   FORWARDING  8000-00:60:cf:46:4f:f0   32769

2     128        0    DISABLED

3     128        5   FORWARDING  8000-00:60:cf:46:4f:f0   32771

4     128        0    DISABLED

5     128        0    DISABLED

6     128        0    DISABLED

7     128        5   FORWARDING  8000-00:60:cf:46:57:c0   32770

8     128        5    BLOCKING   8000-00:60:cf:46:a1:30   32770

9     128        0    DISABLED

VLAN                Name               Status Jumbo BWC  Ports

----  -------------------------------- ------ ----- ---- ----------------

1     Default VLAN                       ena    n   256  1-9

Port  Tag  PVID   BWC        NAME               VLAN(s)

----  ---  ----  -----  -------------  -----------------------

1    n      1    256                   1

2    n      1    256                   1

3    n      1    256                   1

4    n      1    256                   1

5    n      1    256                   1

6    n      1    256                   1

7    n      1    256                   1

8    n      1    256                   1

9    n      1    256                   1

All Trunk groups are disabled.

Forwarding database information:

MAC address     VLAN  Port  State  Referenced ports

-----------------  ----  ----  -----  ----------------

00:60:cf:46:4f:d0     1    7    FWD    7

00:60:cf:46:57:c0     1    7    FWD    7

00:b0:d0:43:76:8f     1    7    FWD    7

Interface information:

1: 10.2.12.1       255.255.255.0   10.2.12.255,     vlan 1, up

2: 10.2.2.1        255.255.255.0   10.2.2.255,      vlan 1, up

3: 10.2.4.1        255.255.255.0   10.2.4.255,      vlan 1, up

Default gateway information: metric strict

Current IP forwarding settings: ON, dirbr disabled

Current local networks:

Current IP port settings:

All other ports have forwarding ON

Current RIP settings:

OFF, update 30

split horizon

BGP Information:

OFF, id 10.2.12.1

BGP Peer Information:

Virtual Router Redundancy is globally turned OFF.

ARP cache information:

IP address    Flags    MAC address    VLAN Port Referenced ports

--------------- ----- ----------------- ---- ---- ----------------

10.2.2.1         P    00:60:cf:46:4f:f0    1       1-9

10.2.2.32             00:60:cf:46:57:c0    1   7   empty

10.2.4.1         P    00:60:cf:46:4f:f0    1       1-9

10.2.4.33             00:60:cf:46:a1:30    1   7   empty

10.2.12.1        P    00:60:cf:46:4f:f0    1       1-9

10.2.12.34            00:b0:d0:43:76:69    1   1   7

10.2.12.35            00:b0:d0:43:76:7f    1   3   7

ARP address information:

IP address        IP mask        MAC address    VLAN Flags

--------------- --------------- ----------------- ---- -----

10.2.4.1        255.255.255.255 00:60:cf:46:4f:f0    1

10.2.2.1        255.255.255.255 00:60:cf:46:4f:f0    1

10.2.12.1       255.255.255.255 00:60:cf:46:4f:f0    1

Route table information:

Destination         Mask          Gateway        Type      Tag    Metr If

--------------- --------------- --------------- --------- --------- ---- --

10.2.1.0        255.255.255.0   10.2.2.32       indirect  static          2

10.2.2.0        255.255.255.0   10.2.2.1        direct    fixed           2

10.2.2.1        255.255.255.255 10.2.2.1        local     addr            2

10.2.2.255      255.255.255.255 10.2.2.255      broadcast broadcast       2

10.2.3.0        255.255.255.0   10.2.4.33       indirect  static          3

10.2.4.0        255.255.255.0   10.2.4.1        direct    fixed           3

10.2.4.1        255.255.255.255 10.2.4.1        local     addr            3

10.2.4.255      255.255.255.255 10.2.4.255      broadcast broadcast       3

10.2.12.0       255.255.255.0   10.2.12.1       direct    fixed           1

10.2.12.1       255.255.255.255 10.2.12.1       local     addr            1

10.2.12.255     255.255.255.255 10.2.12.255     broadcast broadcast       1

127.0.0.0       255.0.0.0       0.0.0.0         martian   martian

224.0.0.0       224.0.0.0       0.0.0.0         martian   martian

255.255.255.255 255.255.255.255 255.255.255.255 broadcast broadcast

Current Bandwidth Management setting: OFF

Current isd server configuration:

0.0.0.0, 0, 0, OFF

isd BOOTP server: OFF

isd master IP: 0.0.0.0

Network interface for isd: 0

Current isd servers:

state   isd IP          mac_addr                revision        life

-----  --------        ----------              ----------      ------

Cannot connect to isd. Please try later.

No isd information yet.

Real server state:

1: 10.2.1.1, 00:60:cf:46:57:c0, vlan 1, port 7, health 3, up

2: 10.2.3.1, 00:60:cf:46:a1:30, vlan 1, port 7, health 3, up

Virtual server state:

Redirect filter state:

224: group 1, health 3, backup none

real servers:

1: 10.2.1.1, backup none, up

2: 10.2.3.1, backup none, up

Port state:

1: 0.0.0.0

filt  enabled, filters: 100 224

2: 0.0.0.0

filt disabled, filters: empty

3: 0.0.0.0

filt  enabled, filters: 100 224

4: 0.0.0.0

filt disabled, filters: empty

5: 0.0.0.0

filt disabled, filters: empty

6: 0.0.0.0

filt disabled, filters: empty

7: 0.0.0.0

filt disabled, filters: empty

8: 0.0.0.0

filt disabled, filters: empty

9: 0.0.0.0

filt disabled, filters: empty

Enabled Software features:

Layer 4: SLB + WCR

14.  ping W.X.Y.Z (where the letters represent an IP address)

[host 10.2.11.32, max tries 5, delay 1000 msec]

10.2.11.32: #1 ok, RTT 0 msec.

10.2.11.32: #2 ok, RTT 1 msec.

10.2.11.32: #3 ok, RTT 1 msec.

10.2.11.32: #4 ok, RTT 1 msec.

10.2.11.32: #5 ok, RTT 1 msec.

Ping finished.

15.  /stat/slb/group 1

------------------------------------------------------------------

Real server group 1 stats:

Current      Total  Highest

Real IP address      Sessions   Sessions Sessions           Octets

---- --------------- -------- ---------- --------  ---------------

1 10.2.11.32             0          0        0                0

2 10.2.11.33             0          0        0                0

---- --------------- -------- ---------- --------  ---------------

0          0        0                0

16.  /stat/port 3/if

------------------------------------------------------------------

Interface statistics for port 3:

ifHCIn Counters       ifHCOut Counters

Octets:                        66215                 774832

UcastPkts:                         0                        452

BroadcastPkts:               533                     6833

MulticastPkts:                   0                      5736

Discards:                           2                            0

Errors:                               0                            0

ifInUnknownProtos:         0

17.  /cfg/slb/cur

Current Layer 4 settings:

ON, direct disabled, matrix enabled, grace disabled

imask 255.255.255.255, pmask 255.255.255.255

mnet 0.0.0.0, mmask 255.255.255.255

Current health scripts settings:

Current real servers settings:

1: 10.2.11.32, enabled, name , weight 1, tmout 10, maxcon 200000

backup none, inter 2, retry 4, restr 8

remote disabled, proxy enabled, submac disabled

real ports:

http: vport http, group 1

virtual server:  1, 10.2.11.11,      enabled

2: 10.2.11.35, enabled, name , weight 1, tmout 10, maxcon 200000

backup none, inter 2, retry 4, restr 8

remote disabled, proxy enabled, submac disabled

real ports:

http: vport http, group 1

virtual server:  1, 10.2.11.11,      enabled

Current virtual servers settings:

1: 10.2.11.11, enabled

virtual ports:

http: rport http, group 1, frags

real servers:

1: 10.2.11.32,      weight 1,  enabled, backup none

2: 10.2.11.35,      weight 1,  enabled, backup none

Current filters:

Current port settings:

1: client  enabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

2: client  enabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

3: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

4: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

5: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

6: client disabled, server disabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

proxy disabled, 0.0.0.0

7: client disabled, server  enabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

8: client disabled, server  enabled, hotstan disabled, intersw disabled

proxy disabled, 0.0.0.0

filt disabled, filters: empty

Current Global SLB settings:

OFF, dns enabled, ttl 60, local disabled, one disabled, alway disabled

http enabled, usern disabled, mincon 1024, inter 1, weight 1, geo enabled

Current remote site settings:

Current Lookup settings:

lookups disabled, dname none

Current internet network preference settings:

Current sync settings:

filt enabled, ports enabled, prios enabled, pips disabled

Current peer switch settings:

APPENDIX B

The following are switch configuration dumps for each exercise:

EXERCISE 1 Basic Switch Configuration

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken  5:55:18 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:4f:d0

/cfg/sys

bootp dis

/cfg/ip/if 1

ena

addr 10.2.11.1

mask 255.255.255.0

broad 10.2.11.255

/cfg/ip/if 2

ena

addr 10.2.1.1

mask 255.255.255.0

broad 10.2.1.255

/cfg/ip/if 3

ena

addr 10.2.3.1

mask 255.255.255.0

broad 10.2.3.255

/

/script end  /**** DO NOT EDIT THIS LINE!

EXERCISE 3 Server Load Balancing

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken  6:47:42 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:4f:d0

/cfg/sys

idle 60

bootp dis

/cfg/stp

off

/cfg/ip/if 1

ena

addr 10.2.11.1

mask 255.255.255.0

broad 10.2.11.255

/cfg/slb

on

/cfg/slb/real 1

ena

rip 10.2.11.32

/cfg/slb/real 1/layer7

/cfg/slb/real 2

ena

rip 10.2.11.33

/cfg/slb/real 2/layer7

/cfg/slb/group 1

metric roundrobin

add 1

add 2

/cfg/slb/virt 1

ena

vip 10.2.11.11

/cfg/slb/virt 1/service http

group 1

/cfg/slb/port 1

client ena

/cfg/slb/port 2

client ena

/cfg/slb/port 7

server ena

/cfg/slb/port 8

server ena

/

/script end  /**** DO NOT EDIT THIS LINE!

EXERCISE 4 VRRP

VRRP-Switch 1

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken 12:06:17 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:4f:d0

/cfg/sys

idle 60

bootp dis

/cfg/stp/brg

prior 32768

hello 2

mxage 6

fwd 4

aging 300

/cfg/ip/if 1

ena

addr 10.2.11.1

mask 255.255.255.0

broad 10.2.11.255

/cfg/vrrp/on

/cfg/vrrp/vr 2

ena

vrid 2

if 1

addr 10.2.11.11

/cfg/slb

on

/cfg/slb/sync/peer 1

ena

addr 10.2.11.2

/cfg/slb/real 1

ena

rip 10.2.11.32

backup 2

/cfg/slb/real 1/layer7

/cfg/slb/real 2

ena

rip 10.2.11.33

/cfg/slb/real 2/layer7

/cfg/slb/group 1

metric roundrobin

health icmp

add 1

/cfg/slb/virt 1

ena

vip 10.2.11.11

/cfg/slb/virt 1/service http

group 1

/cfg/slb/port 1

client ena

/cfg/slb/port 2

client ena

/cfg/slb/port 3

client ena

server ena

/cfg/slb/port 7

server ena

/cfg/slb/port 8

server ena

/

/script end  /**** DO NOT EDIT THIS LINE!

VRRP-Switch 2

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken 12:08:51 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:57:c0

/cfg/sys

bootp dis

/cfg/stp/brg

prior 32768

hello 2

mxage 6

fwd 4

aging 300

/cfg/ip/if 1

ena

addr 10.2.11.2

mask 255.255.255.0

broad 10.2.11.255

/cfg/vrrp/on

/cfg/vrrp/vr 2

ena

vrid 2

if 1

addr 10.2.11.11

/cfg/slb

on

/cfg/slb/sync/peer 1

ena

addr 10.2.11.1

/cfg/slb/real 1

ena

rip 10.2.11.32

/cfg/slb/real 1/layer7

/cfg/slb/real 2

ena

rip 10.2.11.33

backup 1

/cfg/slb/real 2/layer7

/cfg/slb/group 1

metric roundrobin

health icmp

add 2

/cfg/slb/virt 1

ena

vip 10.2.11.11

/cfg/slb/virt 1/service http

group 1

/cfg/slb/port 1

client ena

/cfg/slb/port 2

client ena

/cfg/slb/port 3

client ena

server ena

/cfg/slb/port 7

server ena

/cfg/slb/port 8

server ena

/

/script end  /**** DO NOT EDIT THIS LINE!

EXERCISE 6 Web Cache Redirection

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken  7:19:05 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:4f:d0

/cfg/sys

idle 60

/cfg/ip/if 1

ena

addr 10.2.11.2

mask 255.255.255.0

broad 10.2.11.255

/cfg/slb

on

/cfg/slb/real 1

ena

rip 10.2.11.32

/cfg/slb/real 1/layer7

/cfg/slb/real 2

ena

rip 10.2.11.33

/cfg/slb/real 2/layer7

/cfg/slb/group 1

metric hash

add 1

add 2

/cfg/slb/filt 2

ena

action redir

proto tcp

dport http

group 1

rport 80

/cfg/slb/filt 224

ena

action allow

/cfg/slb/port 2

filt ena

add 2

add 224

/cfg/slb/port 3

filt ena

add 2

add 224

/

/script end  /**** DO NOT EDIT THIS LINE!

EXERCISE 7 FWLB

FWLB Firewall #1

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken 12:51:16 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:57:c0

/cfg/ip/if 1

ena

addr 10.2.1.32

mask 255.255.255.0

broad 10.2.1.255

/cfg/ip/if 2

ena

addr 10.2.2.32

mask 255.255.255.0

broad 10.2.2.255

/cfg/ip/route

add 10.2.11.0 255.255.255.0 10.2.1.1 1

add 10.2.12.0 255.255.255.0 10.2.2.1 2

/cfg/ip/gw 1

ena

addr 10.2.1.1

/cfg/ip/gw 2

ena

addr 10.2.2.1

/

/script end  /**** DO NOT EDIT THIS LINE!

FWLB Firewall #2

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken 12:52:28 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:a1:30

/cfg/ip/if 1

ena

addr 10.2.3.33

mask 255.255.255.0

broad 10.2.3.255

/cfg/ip/if 2

ena

addr 10.2.4.33

mask 255.255.255.0

broad 10.2.4.255

/cfg/ip/route

add 10.2.11.0 255.255.255.0 10.2.3.1 1

add 10.2.12.0 255.255.255.0 10.2.4.1 2

/cfg/ip/gw 1

ena

addr 10.2.3.1

/cfg/ip/gw 2

ena

addr 10.2.4.1

/

/script end  /**** DO NOT EDIT THIS LINE!

FWLB Dirty Side Switch

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken 13:33:31 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:4f:d0

/cfg/sys

bootp dis

/cfg/ip/if 1

ena

addr 10.2.11.1

mask 255.255.255.0

broad 10.2.11.255

/cfg/ip/if 2

ena

addr 10.2.1.1

mask 255.255.255.0

broad 10.2.1.255

/cfg/ip/if 3

ena

addr 10.2.3.1

mask 255.255.255.0

broad 10.2.3.255

/cfg/ip/route

add 10.2.2.0 255.255.255.0 10.2.1.32

add 10.2.4.0 255.255.255.0 10.2.3.33

/cfg/slb

on

/cfg/slb/real 1

ena

rip 10.2.2.1

/cfg/slb/real 1/layer7

/cfg/slb/real 2

ena

rip 10.2.4.1

/cfg/slb/real 2/layer7

/cfg/slb/group 1

metric hash

health icmp

add 1

add 2

/cfg/slb/filt 100

ena

action allow

dip 10.2.11.0

dmask 255.255.255.0

/cfg/slb/filt 224

ena

action redir

group 1

rport 0

/cfg/slb/port 1

filt ena

add 100

add 224

/

/script end  /**** DO NOT EDIT THIS LINE!

FWLB Clean Side Switch

script start "ACEdirector 3" 4  /**** DO NOT EDIT THIS LINE!

/* Configuration dump taken 13:33:15 Tue Feb 13, 2001

/* Version 8.3.22,  Base MAC address 00:60:cf:46:4f:f0

/cfg/sys

bootp dis

/cfg/ip/if 1

ena

addr 10.2.12.1

mask 255.255.255.0

broad 10.2.12.255

/cfg/ip/if 2

ena

addr 10.2.2.1

mask 255.255.255.0

broad 10.2.2.255

/cfg/ip/if 3

ena

addr 10.2.4.1

mask 255.255.255.0

broad 10.2.4.255

/cfg/ip/route

add 10.2.1.0 255.255.255.0 10.2.2.32

add 10.2.3.0 255.255.255.0 10.2.4.33

/cfg/slb

on

/cfg/slb/real 1

ena

rip 10.2.1.1

/cfg/slb/real 1/layer7

/cfg/slb/real 2

ena

rip 10.2.3.1

/cfg/slb/real 2/layer7

/cfg/slb/group 1

metric hash

health icmp

add 1

add 2

/cfg/slb/filt 100

ena

action allow

dip 10.2.12.0

dmask 255.255.255.0

/cfg/slb/filt 224

ena

action redir

group 1

rport 0

/cfg/slb/port 1

filt ena

add 100

add 224

/cfg/slb/port 3

filt ena

add 100

add 224

/

/script end  /**** DO NOT EDIT THIS LINE!

APPENDIX C

SERIAL DOWNLOAD INSTRUCTIONS:  (OPTIONAL)

- Be in your hyperterminal  window.

- Power cycle the switch.

- While the switch is rebooting, hold the shift D (for 38,400) or shift F (56K) on the PC until you see:

“Xmodem flash download 1.0.5

To download to flash use xmodem at 38400 (or 56000 for shift F) baud.

Power cycle to end xmodem.”

- Reconfig your hyperterminal for 38,400/56K baud.

- Go to the Transfer:---Send pull down window on hyperterminal.

- Set the protocol to 1K Xmodem

- Choose the binary file you want to install.

- Press SEND to start install.

- The download will begin….it takes about 5 minutes…progress will be marked by progressing blocks.

- When the blocks stop progressing (note: file size window will disappear when you complete transfer, i.e. window will say 1024k of 1024K), DO NOT CYCLE THE SWITCH.  Wait for the switch to return “DONE”.  It’s verifying the flash.

- After you receive a done, cycle the switch.

- Reset your baud to 9600 baud.

- Kill and restart the hyperterminal.

CONGRATULATIONS!  THIS COMPLETES THIS EXERCISE !!!

STOP

(unless provided additional instructions by the instructor).