SE Config. Notes:

FLB  + SLB Using NAT on Firewalls

All:

Th is document contains config.’s and diagram along with an attachment explaining the double NAT’ing that is taking place.

The DIP is the only thing that gets NAT’ed in most instances; however, it was also necessary to NAT on the SIP.

The main trick, in fact,  was NAT’ing the SIP to the so the clean side switch would know which FW to send the info. back to (thus maintaining state).  Prior to this the SIP was not getting NAT’ed so the switches  didn’t know which FW to send it to.   It chose FW 1 by convention each time.

The Excel file in the attachment explains the process that takes place.

Many thanks to Jack Martyn for presenting two methods for doing this (see FWLBwNAT.zip).  There are two methods, the second one (pps 9-18) is easier to implement and more functional.

My config. is Active-Active 4-Sw. FLB with SLB doing NAT on the FW’s.

NOTES:

- There have been many internal discussions about using this configuration for VPN Load Balancing.  The consensus is that (in theory) it should work.  It should be tested in the lab and verified.

- The main benefit is that the customer can have a server farm with virtually unlimited addressing space.

- It should be noted that this is processor intensive and, thus very taxing to many Firewalls.  As a matter of fact the customer decided to remove it at the last minute for this very purpose.  He was using Nokia FW.’s and after asking Nokia how performance could be improved to compensate for double NAT he was told that this was ill-advised due to processing constraints.  If a customer is using something more powerful such as Sun boxes (or ASIC based architecture sometime in the future) this will not be as much of an issue.

Lew Bachrach

Systems Engineer

Alteon Web Systems

Web Speed for e-Business

www.alteon.com

212/772-0723 (W)

212/772-3322 (F)

800/301-5660 (P)

bachrach@alteon.com