Technical Tip

Configuring NT4.0 server for a basic PPTP connection

TTPPTP.DOC

This document will detail the basic setup and configuration for a PPTP dial in for TCP/IP access to the network.

The document does not address WINS, DHCP, DNS, IPX or NetBios  that may be required  on individual networks.

1.) TCP/IP must be loaded and a valid IP address, subnet and default gateway must be provided by customer.

Start

Settings

Control Panel

NetWork

Protocols

TCP/IP (If TCP/IP Protocol does not appear add it as a new protocol.)

Properties

IP Address

Check "Specify an IP address" enter IP, subnet and Gateway address for the server.

Advanced

Make sure that PPTP filtering is not checked (if this is checked the servers network adapter will reject

all non PPTP packets including PING)

Routing

Check “Enable IP forwarding”

2.) Install Remote Access Services.

Start

Settings

Control Panel

# NetWork

Services

Add

Select  “Remote Access Services”

3.)	Add Point To Point Tunneling Protocol and enter the required number of VPDN up to 256.

Start

Settings

Control Panel

NetWork

Protocols

Add

Select “ Point to Point Tunneling Protocol”

Properties

Set the required number of Virtual Private Networks (One will be required for each active user).

4.) Add VPN for each user and specify a pool of IP address to be used by these dial in users.

Start

Settings

Control Panel

NetWork

Services

# Remote Access Services

Properties

Add VPN for each expected dial in user up to 256.

Network

Select Server settings: TCP/IP

Configure

Check "Entire Network"

Check "Use static address pool" and specify the range of IP addresses to be used for dial in users.

5.) Create valid users with login and passwords as well as rights to services required.

Start

Programs

Administrative Tools

User manager for Domains

User

New User create a new user

Dialin

Check "Grant dial in permission to user"

6.) Client side configuration for Dial up Networking.

# Create a new connection

Right click

Properties

Server Type

Check “Log on to Network”

Check “ TCP/IP”

TCP/IP Settings

Check “Server assigned  IP address”

Check “Use default gateway on remote network”

Note: For NT4.0 Workstation dial up networking, it is not necessary to enable PPTP.