Super textmodes

Terminate will allow you to set up your preferred video mode at startup.

When leaving Terminate, you can reset another mode if you like. Normally

you will reset to 80x25, but you can do whatever you like. If all register

values are zero, nothing will change - so you don't have to reset everytime

you leave Terminate.

SCANMODE in the UTILS\ subdirectory should find all the available

videomodes for you, but some graphics adapters may need some extra

information, such as the ones below:

Video Seven FastWrite VGA & 1024i VGA

Col/Rows  AX   BX

80x25   6F05 0003   (Reset)

80x43   6F05 0040   (Set)

80x60   6F05 0043   (Set)

100x60   6F05 0044   (Set)

132x25   6F05 0041   (Set)

132x28   6F05 0045   (Set)

132x43   6F05 0042   (Set)

TRIDENT TVGA BIOS C.2.1 1024K VGA MODE

Col/Rows AX+BX+CX+DX must be the same

80x25   0003 (Reset)

80x30   0050 (Set)

80x43   0051 (Set)

80x60   0052 (Set)

132x25   0053 (Set)

132x30   0058 (Set)

132x43   0059 (Set)

132x60   0056 (Set)

TSENG ET-4000

Col/Rows  AX

80x25   0003  (Reset)

80x60   0026  (Set)

100x40   002A  (Set)

132x25   0023  (Set)

132x28   0024  (Set)

132x44   0022  (Set)

ATI VGA Wonder, ATI VGA Wonder+

Col/Rows  AX

80x25   0003  (Reset)

132x25   0023  (Set)

132x44   0033  (Set)

OAK OTI-037, 800 x 600

Col/Rows  AX

80x25   0003  (Reset)

132x25   0050  (Set)

132x43   0051  (Set)

132x60   004F  (Set)

XGA

Col/Rows  AX

80x25   0003  (Reset)

132x25   0014  (Set)

Orchid Prodesigner VGA

Col/Rows  AX

80x25   0003 (Reset)

80x60   0026 (Set)

100x40   002A (Set)

132x25   0024 (Set)

132x28   0023 (Set)

132x44   0022 (Set)

Tecmar VGA/AD

Col/Rows  AX

80x25   0003 (Reset)

132x25   0017 (Set)

Paradise VGA & S3

Col/Rows  AX

80x25   0003 (Reset)

132x25   0055 (Set)

132x43   0054 (Set)