MODEM SETTINGS INFORMATION

If you want your own modem to be included inside the TERMINAT.MOD modem data

file, you can create a file containing data about how to setup your modem.

Please send your modem information to your nearest registration site and

we will make sure it is included with the next version of Terminate.

Line  Description

����  ��������������������������������������������������������������������

1   Name that will be shown in menu

2   Baudrate to use

3   Initstring 1

4   Dial prefix 1

5   Dial suffix 1

6   Connect string

7   No connect string 1

8   No connect string 2

9   No connect string 3

10   No connect string 4

11   Dial cancel string

12   Hangup string  DTR = Toggle DTR, else use string like ATH0

13   Ring string

14   Auto-answer string

15   Ok string

16   CTS/RTS hardware flow control   0=Off  1=On

17   Xon/Xoff software flow control  0=Off  1=On

18-  Lines starting with:

$ Information shown before sending commands to modem

^ Commands to send to modem

* Information shown after sending commands to modem

You can use up to 10 lines for the $,^,* statements, but they MUST come in

the order specified above, see the following example.

Just use this as a model and then change the options you need, removing

the line and numbers to the right:

Line

----

@ZyXEL U-1496, U-1496B, U-1496E V.32bis V.42bis                           1

57600                                                                     2

ATZ^M                                                                     3

ATDT                                                                      4

^M                                                                        5

CONNECT                                                                   6

NO DIAL TONE                                                              7

NO CARRIER                                                                8

BUSY                                                                      9

VOICE                                                                    10

^M                                                                       11

DTR                                                                      12

RING                                                                     13

ATA^M                                                                    14

OK                                                                       15

1                                                                        16

0                                                                        17

$There is no switch to set, just let the program write the

$setting to V-RAM.  If you do not have a 16650 UART installed in

$your I/O card, please use 38400 buad for reliable operation.

^AT&F^M

^ATQ0M1X6V1S7=60^M

^AT&K4&B1&C1&D2&H3*E0&N0&W^M

*Your ZyXEL is now configured.  To get the most out of it,

*be sure to set all your dialing directory entries to 57600 baud.