AVB V2 Stream Reservation Capabilities

May 17, 2012

Andre Fredette

Extreme Networks

- Introduction

The purpose of this document is to provide a list of the capabilities that have been discussed as possible goals of future work for AVB stream reservation.  It is not intended to be a prioritized list, nor is it intended to provide solutions for these capabilities.  Furthermore, the capabilities may be delivered in multiple ways including the following:

  - Set up by protocol.

    - SPB-V/M

    - SRP Extensions/Optimizations

    - Other.

  - “Traffic engineered” w/o protocol.

    - Computed by off-line tool (or human).

    - Configure via SNMP or other means.

  - Some combination of the above.

- Capabilities:

  - Multi-path (Availability).

    - Primary/Backup for fast fail-over.

      - “Active-Passive”

      - Can’t have topology algorithm interfere w/backup path.

    - Simultaneous transmission over multiple paths.

      - “Active-Active”.

      - “1+1 redundancy”.

      - Listener receives multiple copies of each frame and decides which to use.

      - Immediate/Lossless recovery.

      - Can’t have topology algorithm interfere w/backup path.

  - Multipath (Use more of the network).

    - Use more of the network than just a single spanning tree (e.g., MSTP/SPB)

    - Allow the use of more of network than just the shortest path.

      - Find a route even if resources are not available on the shortest path.

      - Dynamically determine best route.

    - Multiple active paths for increased bandwidth.

      - As in MLPP or LAG.

  - Pick the “best” path

    - Reduce latency by using “shortest path” from talker to listener.

    - Constraint-based routing.

      - Less jitter.

      - Less packet loss

      - Lower bandwidth variance on dynamic network.

      - Etc.

  - Solve scalability issues w/ MSRP

    - Chatty protocol.

    - Lot’s of information exchanged on a regular basis.

    - Needs to scale better.

    - Needs to recover faster/more predictably.

- Next Steps.

  - Use Cases per Market.

    - Home network

    - Automotive

    - Industrial

    - Pro AV

  - Pick which problems to solve.

  - Key Questions to Answer.

    - What does this mean for the FDB Configuration?

    - What higher-layer protocols exist or are needed to make use of these facilities?