|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  |  | SG15-LS188 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | STUDY GROUP 15 |  |
|  |  |  |  |  | Original: English |  |
| Question(s): |  |  | 14/15 |  | Geneva, 1-12 July 2019 |  |
| LS |  |  |  |  |  |  |
| Source: |  |  | ITU-T Study Group 15 |  |  |  |
| Title: |  |  | LS on coordination on information and data modelling |  |  |  |
| LIAISON STATEMENT |  |  |  |  |  |  |
| For action to: |  |  |  | IEEE 802.1 Working Group |  |  |
| For comment to: |  |  |  |  |  |  |
| For information to: |  |  |  | IEEE 802.3 Working Group, ONF, MEF, IETF, BBF |  |  |
| Approval: |  |  |  | ITU-T SG15 (12 July 2019) |  |  |
| Deadline: |  |  |  | 10 September 2019 |  |  |
| Contact: |  | Hing-Kam Lam Rapporteur Q14/15 |  |  |  | Tel: +1 732-275-4646 Email: kamlam@fiberhome.com |
| Contact: |  | Scott Mansfield Associate Rapporteur Q14/15 |  |  |  | Tel: +1 613-963-6171 Email: scott.mansfield@ericsson.com |

ITU-T Q14/15 has been working on modelling of Ethernet OAM to augment the IEEE 802.1Qcx CFM with the ITU-T G.8013 OAM. The CFM functions between the IEEE 802.1Qcx YANG modules and ITU-T G.8051 and G.8052 managed MI signals have been analysed. The followings are a summary of the analysis. It would benefit from setting default values for the following attributes in the CFM YANG modules.

- Continuity Check

Some IEEE specific operations are not supported in ITU.

- rmep-failed-ok-time, mac-address in remote mep-db: These are read-only parameters, so it is possible to omit them.

- Port Status TLV, Interface Status TLV, Sender ID TLV: G.8052.1 can exclude them because they are optional TLVs.

One ITU specific operation is not supported in IEEE.

- Proactive LM: It needs to be augmented in G.8052.1 as the part of performance monitoring functions.

The definition of defect detection is different between IEEE and ITU.

- cDEG, cUNPr: Not supported in 802.1Q

- xconCCMdefect:  Composition of cUNL and cMMG

- errorCCMdefect: Composition of cUNP and cUNM

- FNG(Fault Notification Generator): Not specified in G.8051

- Loopback

Some ITU specific operations are not covered in P802.1Qcx.

- Series: Configuration of the LBM Tx period and collection of total number of LBM frames.

They should be supported with the augmentation approach suggested in P802.1Qcx.

- Two-way Test: Support of Test TLV (including the number of CRC/BER/OO frames) and running infinite number of times.  It needs to be augmented in G.8052.1 as the part of performance monitoring functions.

Some IEEE specific parameters are not supported in ITU.

- lbm-dest-mep-id:  G.8052.1 has only to exclude this operation because it is optional in 802.1Q.

- Lbm-request-id: Since they are read-only parameters, there is no impact if they would be pruned in G.8052.1.

- Linktrace

Some IEEE specific operations are not supported in ITU.

- ltm-target-mep-id: G.8052.1 has only to exclude this operation because it is optional in 802.1Q.

- ltm-flags, ltm-transaction-id, ltm-egress-identifier, ltr-receive-order, ltr-forwarded, lter-terminal-mep, ltr-relay: Since they are read-only parameters, there is no impact if they would be pruned in G.8052.1.

The result of the detailed comparison is shown in the Excel file:

ITU-T Q14/15 has been hosting regular virtual meetings since November 2018 on coordination of information/data modelling on topics such as Ethernet OAM. Experts of SDOs, including IEEE 802.1Qcx, have been invited to participate in these meetings.

ITU-T Q14/15 will continue to host coordination virtual meetings on the following dates. Modelling experts of the receiving SDOs of this liaison statement are invited to join.

Logistics details:

- Dates:	2019 September 9; November 11; 2020 January 13

- Time:	3:00 PM - 4:00 PM CET (Geneva Switzerland)

- URL:	Series of virtual meetings at https://global.gotomeeting.com/join/897704405

- Access Code: 897-704-405

- Dial in:	United States: +1 (312) 757-3117